SECURITY INCIDENT REPORT Honeypot Deployment & Network Attack Analysis Lab Environment ¦ Cybersecurity Portfolio

| Report Title | Honeypot Deployment & Network Attack Detection |
| --- | --- |
| Date | March 4, 2026, Uploaded April 27, 2026 |
| Prepared By | ADIGUN |
| Course | Cybersecurity --PROJECT 1 |
| Environment | Controlled Lab (UTM VMs on macOS Apple Silicon) |
| Attacker IP | 172.20.10.9 (Kali Linux) |
| Target IP | 172.20.10.7 (Ubuntu, Cowrie Honeypot) |
| Classification | Academic / Portfolio |

1. Executive Summary

This report documents a simulated cyber attack conducted in a controlled two-VM lab environment. A Cowrie SSH honeypot was deployed on an Ubuntu machine to detect, log, and analyse unauthorized access attempts. An attacker machine running Kali Linux performed reconnaissance and brute force attacks against the honeypot.

The honeypot successfully intercepted and logged all attack activity including port scanning, SSH probing, credential brute forcing, and a 3-minute interactive session. This report details the full attack timeline, technical findings, log evidence, and recommended defences.

2. Lab Environment

2.1 Infrastructure

Both virtual machines were deployed using UTM on macOS Apple Silicon (aarch64 architecture).

| Component | Role | IP Address | OS |
| --- | --- | --- | --- |
| Ubuntu VM | Honeypot / Defender | 172.20.10.7 | Ubuntu Linux (aarch64) |
| Kali Linux VM | Attacker | 172.20.10.9 | Kali Linux 2024 |

2.2 Tools Used

| Tool | Purpose | Machine |
| --- | --- | --- |
| Cowrie SSH Honeypot | Trap and log attack activity | Ubuntu (Defender) |
| iptables | Redirect port 22 → 2222 | Ubuntu (Defender) |
| Nmap 7.95 | Port scanning & service detection | Kali (Attacker) |
| Wireshark / tcpdump | Capture network traffic (.pcap) | Kali (Attacker) |
| OpenSSH Client | Manual SSH login attempt | Kali (Attacker) |

3. Network Protocol Analysis

3.1 Primary Protocol -SSH (Secure Shell)

The primary protocol involved in this incident was SSH (Secure Shell) operating on TCP Port 22. SSH is a cryptographic network protocol designed for secure remote login and command execution over unsecured networks. It uses asymmetric key exchange during session setup and symmetric encryption for data transfer.

In this incident, the attacker used SSH to probe, authenticate, and gain access to the honeypot. All SSH traffic was intercepted by Cowrie before reaching the real system.

3.2 Protocol Stack Observed

| Layer | Protocol | Detail |
| --- | --- | --- |
| Application | SSH-2.0 | OpenSSH / Nmap-SSH2-Hostkey / Cowrie emulated server |
| Transport | TCP | Port 22 (redirected to 2222 via iptables PREROUTING) |
| Network | IPv4 | 172.20.10.9 → 172.20.10.7 |
| Key Exchange | curve25519-sha256 | Used in successful session; Nmap attempted diffie-hellman-group14-sha1 |
| Encryption | aes128-ctr | With hmac-sha2-256 for integrity |

4. Attack Timeline

The attack was conducted in two distinct phases: reconnaissance via Nmap scanning, followed by targeted SSH brute force and interactive access.

| Time (UTC) | Phase | Event |
| --- | --- | --- |
| 15:21:14 | Reconnaissance | Nmap -A -T4 scan initiated on ports 22 and 23 |
| 15:21:15 | Reconnaissance | Scan completed in 1.90s -Port 22 OPEN, Port 23 CLOSED |
| 15:28:48 | SSH Probing | Nmap SSH hostkey scripts fired -multiple rapid connections |
| 15:28:48 | SSH Probing | Cowrie fingerprinted Nmap: SSH-2.0-Nmap-SSH2-Hostkey / HASSH: e788c657 |
| 15:28:48 | SSH Probing | KEX negotiation failed on several probes (couldn't match kex parts) |
| 15:28:54 | Brute Force | SSH connection from OpenSSH_10.0p2 -public key auth attempted and FAILED |
| 15:28:57 | Brute Force | Password attempt: root / 12345 -LOGIN SUCCEEDED |
| 15:28:57 | Post-Access | Cowrie spawned emulated Linux shell (linux-x64-lsb) |
| 15:31:57 | Session End | Attacker disconnected -session duration: 183.5 seconds |

5. Nmap Scan Results

5.1 Scan Command

sudo nmap -A -T4 -p 22,23 172.20.10.7 -oA /home/kepler/cowrie_results/cowrie_nmap

5.2 Findings

| Port | State | Service | Version / Detail |
| --- | --- | --- | --- |
| 22/tcp | OPEN | SSH | OpenSSH 9.2p1 Debian 2+deb12u3 (protocol 2.0) |
| 23/tcp | CLOSED | Telnet | Not available |

5.3 OS & Network Detection

| Detected OS | Linux 4.15 - 5.19 / MikroTik RouterOS 7.x |
| --- | --- |
| MAC Address | 1A:EA:60:16:4E:2D |
| Network Distance | 1 hop (direct LAN connection) |
| Latency | 0.60ms |
| Scan Duration | 1.90 seconds |
| SSH Host Keys | RSA 2048-bit, ECDSA 256-bit, ED25519 256-bit |

5.4 Observation

Nmap detected the real OpenSSH banner (9.2p1) during the -A scan because the OS/version detection phase connects directly. However, Cowrie's logs confirm it intercepted the traffic and logged the Nmap fingerprint, confirming the iptables redirect was active.

6. Cowrie Honeypot Log Evidence

6.1 Key Log Entries

New connection: 172.20.10.9:59586 (172.20.10.7:2222) [session: bb8c5046a737]

Remote SSH version: SSH-2.0-OpenSSH_10.0p2 Debian-5

SSH client hassh fingerprint: eeca2460550b9ded084ecf2f70a75356

public key login attempt for [root] failed

login attempt [root/12345] succeeded

Initialized emulated server as architecture: linux-x64-lsb

Connection lost after 183.5 seconds

6.2 Notable Findings from Logs

- Cowrie correctly identified Nmap using its HASSH fingerprint (e788c657d1a22971d5026526ffd2e918)

- Nmap's SSH probes were rejected due to KEX algorithm mismatch -expected behaviour

- The real SSH login used OpenSSH_10.0p2 from Kali -different from Nmap's probe client

- Public key authentication was attempted first and failed before password was tried

- Weak password '12345' allowed successful login -critical vulnerability

- Cowrie activated its default credential database as etc/userdb.txt was not configured

- Attacker remained in the fake shell for 183 seconds -all commands would have been logged

7. Wireshark Capture Analysis

7.1 Capture Details

| Capture File | cowrie_capture.pcap |
| --- | --- |
| File Size | 304 KB |
| Captured On | Kali Linux (attacker interface) |
| Filter Used | host 172.20.10.7 |
| Interface | eth0 |
| Capture Period | Full attack duration |

7.2 Traffic Patterns Observed

- SYN packets from Kali to Ubuntu on ports 22 and 23 -Nmap TCP SYN scan

- SSH handshake (TCP three-way handshake) followed by SSH banner exchange

- Multiple rapid connections during Nmap SSH script phase

- Sustained SSH session traffic for 183 seconds after successful login

- TCP FIN/RST packets marking session termination

7.3 Recommended Wireshark Filters

tcp.port == 22                          # All SSH traffic

ip.addr == 172.20.10.7                  # All traffic to honeypot

tcp.flags.syn == 1 && tcp.flags.ack == 0  # Nmap SYN probes

ssh                                     # SSH protocol only

8. Vulnerabilities Identified

| # | Vulnerability | Severity | Detail |
| --- | --- | --- | --- |
| 1 | Weak SSH Password | CRITICAL | root/12345 accepted -trivially guessable |
| 2 | Root Login Enabled | HIGH | SSH allows direct root authentication |
| 3 | No Rate Limiting | HIGH | No lockout after repeated failed attempts |
| 4 | Port 22 Exposed | MEDIUM | Default SSH port is a known attack target |
| 5 | userdb.txt Missing | LOW | Cowrie fell back to default credentials database |

9. Remediation Recommendations

9.1 Implement Fail2Ban (Brute Force Protection)

Fail2Ban monitors authentication logs and automatically bans IP addresses that exceed a threshold of failed login attempts. This directly addresses the brute force vector used in this attack.

sudo apt install fail2ban

sudo nano /etc/fail2ban/jail.local

Add the following configuration:

[sshd]

enabled  = true

port     = 22

maxretry = 3

bantime  = 3600

findtime = 600

Effect: After 3 failed attempts within 10 minutes, the attacker IP is banned for 1 hour.

9.2 Disable Root SSH Login

sudo nano /etc/ssh/sshd_config

PermitRootLogin no

sudo systemctl restart ssh

9.3 Enforce SSH Key Authentication Only

PasswordAuthentication no

PubkeyAuthentication yes

9.4 Change Default SSH Port

Port 2222   # in /etc/ssh/sshd_config

Reduces automated scanning noise significantly as most bots target port 22 only.

9.5 Summary Table

| Recommendation | Addresses | Priority |
| --- | --- | --- |
| Deploy Fail2Ban | Brute force / rate limiting | IMMEDIATE |
| Disable root login | Privilege escalation risk | IMMEDIATE |
| SSH keys only | Password-based attacks | HIGH |
| Change SSH port | Automated scanning | MEDIUM |
| Configure userdb.txt | Honeypot credential logging | LOW |

10. Conclusion

This lab successfully demonstrated the full attack-detect-respond cycle in a controlled environment. The Cowrie SSH honeypot effectively intercepted and logged all attacker activity -from initial Nmap reconnaissance through to an interactive session -without the attacker ever touching the real system.

Key outcomes achieved:

- Cowrie honeypot deployed and operational on Ubuntu with iptables port redirection

- Nmap scan successfully fingerprinted by Cowrie using HASSH analysis

- Brute force attack simulated and captured in both Cowrie logs and Wireshark pcap

- Successful fake login with weak credentials (root/12345) demonstrated real-world risk

- Full network traffic captured in .pcap format for forensic analysis

The exercise highlights how honeypots serve as a powerful early-warning system and threat intelligence tool, providing defenders with detailed attacker behaviour data while protecting real infrastructure.

Appendix -Key Files

| File | Location | Contents |
| --- | --- | --- |
| cowrie_nmap.nmap | /home/kepler/cowrie_results/ | Nmap scan output -human readable |
| cowrie_nmap.gnmap | /home/kepler/cowrie_results/ | Nmap scan output -grepable format |
| cowrie_capture.pcap | /home/kepler/ | Full Wireshark packet capture (304KB) |
| cowrie.log | ~/cowrie/var/log/cowrie/ | Full Cowrie honeypot event log |